| NORMAL PROCEDURES PA-28-181 PIPER PILOT |  |  |  | NORMAL PROCEDURES PA-28-181 PIPER PILOT |  |
| --- | --- | --- | --- | --- | --- |
| PREFLIGHT CHECKLIST COCKPIT TIE DOWN & PITOT COVER REMOVE AIRCRAFT DOCUMENTS AROW PARKING BRAKE SET ALL INSTRUMENT PANEL SWITCHES OFF MIXTURE IDLE CUT-OFF LEFT/RIGHT MAGS OFF BATTERY MASTER ON PANEL LIGHTING VERIFY OPERATION PITOT HEAT ON EXTERIOR LIGHTING ON & VERIFY OPERATIONS STALL WARNING TONE CHECK PITOT/STATIC HEAD CHECK – WARM *CAUTION* PITOT HEAD CAN BE VERY HOT DO NOT OPERATE LONGER THAN 3 MIN SEE POH FOR FULL CAUTION MESSAGE P HEAT OFF CAS MESSAGE EXTINGUISHED FUEL CHECK QUANTITY & IMBALANCE ALL LIGHTING OFF PITOT HEAT OFF P HEAT OFF CAS MESSAGE ILLUMINATED BATTERY MASTER OFF FLAPS EXTEND PRIMARY FLIGHT CONTROLS PROPER OPERATION STABILATOR AND RUDDER TRIM NEUTRAL ALTERNATE STATIC CHECK PITOT & STATIC SYSTEMS DRAIN WINDOWS CHECK, CLEAN BAGGAGE SECURE RIGHT WING SURFACE CONDITION CLEAR OF ICE/FROST/SNOW FLAP AND HINGES NO DAMAGE OR INTERFERENCE AILERON AND HINGES NO DAMAGE OR INTERFERENCE STATIC WICKS (3) CHECK SECURE WING TIP AND LIGHTS CHECK FUEL TANK SUMP DRAIN, CHECK FOR WATER, SEDIMENT, AND PROPER FUEL FUEL TANK VENT CLEAR MAIN GEAR STRUT PROPER INFLATION (4.5 ± .25IN.) TIRE CHECK BRAKE BLOCK AND DISCS CHECK FUEL TANK CHECK SUPPLY, SECURE CAP FRESH AIR INLET CLEAR |  |  |  | NOSE SECTION GENERAL CONDITION CHECK VENT FAN INLET CLEAR COWLING SECURE WINDSHIELD CLEAN OIL CHECK LEVEL DIPSTICK PROPERLY SEATED & SECURED OIL FILLER DOOR SECURE PROPELLER AND SPINNER CHECK AIR INLETS (4) CLEAR NOSE CHOCK REMOVE NOSE GEAR STRUT PROPER INFLATION (3.25 ± .25 IN.) NOSE TIRE CHECK FUEL STRAINER DRAIN LEFT WING SURFACE CONDITION CLEAR OF ICE/FROST/SNOW FRESH AIR INLET CLEAR MAIN GEAR STRUT PROPER INFLATION (4.5 ± .25 IN.) TIRE CHECK BRAKE BLOCK AND DISC CHECK FUEL TANK SUMP DRAIN, CHECK FOR WATER, SEDIMENT, AND PROPER FUEL FUEL TANK VENT CLEAR FUEL TANK CHECK SUPPLY, SECURE CAP PITOT/STATIC HEAD CLEAR OAT PROBE CHECK WING TIP AND LIGHTS CHECK AILERON AND HINGES NO DAMAGE OR INTERFERENCE FLAP AND HINGES NO DAMAGE OR INTERFERENCE STATIC WICKS (3) CHECK SECURE FUSELAGE ANTENNAS CHECK EMPENNAGE CLEAR OF ICE/FROST/SNOW STABILATOR AND TRIM TAB NO DAMAGE OR INTERFERENCE RUDDER NO DAMAGE OR INTERFERENCE STATIC WICKS (3) CHECK SECURE ELT PLATE SECURE EXTERNAL POWER DOOR SECURE WALK AROUND AIRCRAFT NOTE GENERAL CONDITION |  |
| University of Oklahoma School of Aviation Printed 11/18/2024 |  | 1 |  | University of Oklahoma School of Aviation Printed 11/18/2024 | 2 |
| NORMAL PROCEDURES PA-28-181 PIPER PILOT |  |  |  | NORMAL PROCEDURES PA-28-181 PIPER PILOT BEFORE ENGINE START THROUGH ENGINE SHUTDOWN – CHECKLIST WILL BE VERBALIZED ENGINE RUN-UP (GROUND CHECK) PARKING BRAKE SET THROTTLE 2000 RPM LEFT/RIGHT MAG CHECK CHECK L & R (MAX DROP 175 & MAX DIFF 50 RPM) OIL TEMPERATURE CHECK OIL PRESSURE CHECK VOLTS INDICATION CHECK BUS (28 ±1 VOLT) ALT AMPS INDICATION CHECK NORMAL ALT. AIR CHECK FOR RPM DROP FUEL PUMP OFF, VERIFY ENGINE OPERATION THROTTLE IDLE CHECK, 1000 RPM FLIGHT CONTROLS FREE & CORRECT ELECTRIC TRIM TEST AUTOPILOT ENGAGE VERIFY AP CAN BE OVERPOWERED A/P DISC BUTTON PRESS, VERIFY AP DISENGAGES AND AUDIO ALERT TRIM NEUTRAL CABIN DOOR CLOSED AND SECURE |  |
| BEFORE ENGINE START THROUGH ENGINE SHUTDOWN – CHECKLIST WILL BE VERBALIZED BEFORE ENGINE START MX TIME CHECK TIME REMAINING VS TIME ON CHECKLIST COVER BILLING TIME/HOBBS TIME RECORD FLAPS RETRACT CABIN DOOR CLOSED AND SECURE SEATS ADJUSTED AND LOCKED IN POSITION SEATBELTS AND HARNESSES FASTEN AND SECURE PARKING BRAKE SET PASSENGER BRIEF COMPLETE ENGINE START CIRCUIT BREAKERS IN ALT. AIR CLOSE ALTERNATE STATIC SOURCE OFF FUEL SELECTOR CYCLE, FULLEST TANK MASTER ON ALTERNATOR ON DATABASES VERIFY CURRENCY AUTOPILOT VERIFY PREFLIGHT SELF-TEST COMPLETED LEFT/RIGHT MAG SWITCHES ON ANTI-COLLISION LIGHTS ON AREA CLEAR THROTTLE OPEN 1/8 INCH FUEL PUMP ON MIXTURE COLD START GPH ~4.5 THEN IDLE CUT-OFF HOT START IDLE CUT-OFF STARTER ENGAGE MIXTURE ADVANCE THROTTLE 1000 RPM OIL PRESSURE CHECK PRE-TAXI CAS MESSAGE (G3X) CONSIDER ANY ILLUMINATED PFD ANNUNCIATIONS NO RED X’S FUEL CALC TAB (ENG PAGE) ENTER FUEL QUANTITY SYSTEM MESSAGES CONSIDER LIGHTS AS REQUIRED VENTILATION, HEATER, AND DEFROSTER AS DESIRED COM/NAV RADIOS & AVIONICS CHECK & SET STANDBY FLIGHT INSTRUMENT VERIFY ON WITH NO RED-X’S OR FAILURE ANNUNCIATIONS AWOS/ATIS CHECK ALTIMETER SET, VERIFY BOTH TRANSPONDER LOCAL CODE & ALT PARKING BRAKE RELEASE BRAKES TEST |  |  |  |  |  |
| University of Oklahoma School of Aviation Printed 11/18/2024 |  | 3 |  | University of Oklahoma School of Aviation Printed 11/18/2024 | 4 |
| NORMAL PROCEDURES PA-28-181 PIPER PILOT |  |  |  | NORMAL PROCEDURES PA-28-181 PIPER PILOT |  |
| BEFORE ENGINE START THROUGH ENGINE SHUTDOWN – CHECKLIST WILL BE VERBALIZED BEFORE TAKE-OFF TYPE OF TAKEOFF DETERMINE AIRSPEEDS: ROTATION, CLIMB OUT, AND BEST GLIDE BRIEF PRE-TAKEOFF BRIEFING COMPLETE FLAPS SET FUEL PUMP ON FLIGHT INSTRUMENTS NO RED X’S STANDBY FLIGHT INSTRUMENT NO RED X’S CAS MESSAGE (G3X) CONSIDER ANY ILLUMINATED SYSTEM MESSAGES CONSIDER ENGINE INDICATIONS CHECK ANTI-COLLISION LIGHTS ON LANDING LIGHTS ON MIXTURE RICH BELTS/HARNESS FASTENED & CHECK STABILATOR/RUDDER TRIM SET CABIN DOOR LATCHED TAKE-OFF TIME RECORD VS0 – 45 KIAS VS1 – 50 KIAS VR – 60 KIAS VX – 64 KIAS VY – 76 KIAS Best Glide – 76 KIAS Cruise Climb – 87 KIAS VFE – 102 KIAS VO – 113 KIAS VNO – 125 KIAS VNE – 154 KIAS |  |  |  | BEFORE ENGINE START THROUGH ENGINE SHUTDOWN – CHECKLIST WILL BE VERBALIZED CLIMB (BETWEEN 500 AND 1500 AGL) POWER SET TRIM SET FUEL PUMP OFF LANDING LIGHTS OFF CRUISE (AT TOP OF CLIMB FOR X/C AND LCL FLIGHTS) POWER SET TRIM SET MIXTURE AS REQUIRED TRUE AIR SPEED (X/C FLTS) VERIFY IN-RANGE (X/C – BEFORE TOP OF DESCENT; LCL – BEFORE TURNING TO OUN) COM/NAV RADIOS & AVIONICS CHECK & SET AWOS/ATIS CHECK ALTIMETER SET, VERIFY BOTH SEATBELTS FASTEN AND SECURE MIXTURE RICH FUEL PUMP ON FUEL SELECTOR FULLEST TANK LANDING LIGHTS ON APPROACH AND LANDING (BEFORE PATTERN ENTRY AND BEFORE MIDFIELD DOWNWIND EACH SUBSEQUENT APPROACH) TYPE LANDING DETERMINE FLAPS SET (102 KIAS MAX.) ALT. AIR AS REQUIRED AUTOPILOT DISCONNECT (ABOVE 200AGL) INITIAL APPROACH SPEED 75-95 KIAS FINAL APPROACH SPEED 66 KIAS GO-AROUND MIXTURE FULL RICH THROTTLE FULL OPEN CONTROL WHEEL BACK PRESSURE TO OBTAIN POSITIVE CLIMB ATTITUDE FLAPS RETRACT INCREMENTALLY |  |
| University of Oklahoma School of Aviation Printed 11/18/2024 |  | 5 |  | University of Oklahoma School of Aviation Printed 11/18/2024 | 6 |
| NORMAL PROCEDURES PA-28-181 PIPER PILOT |  |  |  | NORMAL PROCEDURES PA-28-181 PIPER PILOT |  |
| BEFORE ENGINE START THROUGH ENGINE SHUTDOWN – CHECKLIST WILL BE VERBALIZED AFTER LANDING (AFTER CLEARING RUNWAY) FLAPS RETRACT FUEL PUMP OFF LANDING LIGHTS AS REQUIRED TRANSPONDER LOCAL CODE & ALT |  |  |  | BEFORE ENGINE START THROUGH ENGINE SHUTDOWN – CHECKLIST WILL BE VERBALIZED ENGINE SHUTDOWN PARKING BRAKE RELEASED FLAPS VERIFY RETRACTED FUEL PUMP VERIFY OFF ELECTRICAL SWITCHES OFF ALTERNATOR OFF THROTTLE IDLE MIXTURE IDLE CUT-OFF LEFT/RIGHT MAG SWITCHES OFF INTERIOR LIGHTS OFF EXTERIOR LIGHTS OFF BATTERY MASTER OFF BILLING/MX TIME RECORD POST-FLIGHT - PUT SUN VISOR IN WINDOW (IN SUMMER) - GATHER PERSONAL BELONGINGS – NO TRASH LEFT - PLACE YELLOW PLACARD -- IF MX REQUIRED - SECURE COCKPIT DOOR (TOP LATCH IF RAINING SOON) - LOCK AIRCRAFT DOOR - WALK AROUND AIRCRAFT – LOOK AT CONDITION STRAP DOWN (3 STRAP POINTS) OR CHOCK (IF NO STRAP AREA AVAILABLE CROSS-COUNTRY) IF NECESSARY - CLOSE FLIGHT PLAN - TURN IN AIRCRAFT KEY TO DISPATCH OR NIGHT MX - FILL OUT ANY SAFETY REPORTS ONLINE - REPORT SQUAWKS OR ANY DISCREPANCY TO DISPATCH OR NIGHT MX - TURN IN FUEL CARD WITH RECEIPT (IF USED) - FILL OUT YOUR “PERSONAL LOGBOOK WITH FLIGHT TIME/DETAILS” ABNORMAL PROCEDURES HARD LANDING (TAXI TO MX AND INFORM AVIATION STAFF) UNCONTROLLED DEPARTURE FROM PAVEMENT (RUNWAY OR TAXIWAY) - DO NOT ATTEMPT TO TAXI BACK ONTO PAVEMENT - RUN ENGINE SHUTDOWN CHECKLIST - CONTROLLED FIELD: NOTIFY TOWER AND ADVISE AIRCRAFT MUST BE TOWED - UNCONTROLLED FIELD: CALL OU MOBILE PHONE AND INSTRUCTOR SYMPTOMS OF VAPOR LOCK - FLUCTUATION OF IDLE SPEED AND FUEL FLOW - POOR ENGINE RESPONSE TO THROTTLE MOVEMENT - ENGINE WILL NOT OPERATE WHEN THROTTLE IS CLOSED - HIGH RPM DROP (GREATER THAN 175 RPM) DURING MAG CHECK CORRECTIVE ACTION FOR VAPOR LOCK - ADVANCE THROTTLE TO ~2000 RPM FOR 1-2 MIN OR UNTIL SMOOTH ENGINE OPERATIONS - RETARD THROTTLE TO IDLE AND CHECK FOR SMOOTH OPERATION - DURING TAXI, LEAN MIXTURE AND OPERATE AT A HIGH POWER SETTING (MAX 1200 RPM) AS PRACTICAL - PRIOR TO TAKEOFF SET MIXTURE TO FULL RICH - VERIFY SMOOTH ENGINE OPERATION DURING TAKEOFF ROLL |  |
| University of Oklahoma School of Aviation Printed 11/18/2024 |  | 7 |  | University of Oklahoma School of Aviation Printed 11/18/2024 | 8 |
| EMERGENCY PROCEDURES PA-28-181 PIPER PILOT |  |  |  | EMERGENCY PROCEDURES PA-28-181 PIPER PILOT |  |
| ENGINE FIRE DURING START STARTER CONTINUE TO CRANK ENGINE MIXTURE IDLE CUT-OFF THROTTLE OPEN FUEL PUMP OFF FUEL SELECTOR OFF IF FIRE CONTINUES EVACUATE AIRCRAFT USE FIRE EXTINGUISHER IF AVAILABLE ENGINE FIRE IN FLIGHT FUEL SELECTOR OFF THROTTLE CLOSED MIXTURE IDLE CUT-OFF FUEL PUMP OFF HEAT/DEFROSTER OFF DEFROSTER OFF VENTS OPEN IF FIRE CONTINUES AIRSPEED INCREASE IN ATTEMPT TO BLOW OUT FIRE ENGINE SECURE ENGINE SECURE IN FLIGHT FUEL SELECTOR OFF LEFT/RIGHT MAGS OFF THROTTLE CLOSED MIXTURE IDLE CUT-OFF SEAT BELTS TIGHTEN DOOR UNLATCH ELT ON *PREPARE FOR EMERGENCY LANDING (66 KNOTS SHORT FINAL) AFTER TOUCHDOWN MASTER/ALTERNATOR OFF ELECTRICAL FIRE MASTER OFF ALTERNATOR OFF VENTS OPEN HEAT/DEFROSTER OFF FIRE EXTINGUISH EMERGENCY DESCENT (IF NEEDED) TO A SAFE ALTITUDE *LAND ASAP |  |  |  | ENGINE ROUGHNESS ALT. AIR OPEN IF ROUGHNESS CONTINUES AFTER 1 MIN MIXTURE ADJUST FOR MAX SMOOTHNESS ALT. AIR CLOSE FUEL PUMP ON FUEL SELECTOR SWITCH TANKS ENGINE INDICATORS CHECK LEFT/RIGHT MAGS INDIVIDUALLY SELECT OFF AND ON *LAND ASAP ENGINE FAILURE/PARTIAL POWER LOSS PITCH (BEST GLIDE) 76 KTS TRIM 76 KTS FIELD SELECT SUITABLE FUEL SELECTOR SWITCH TANKS LEFT/RIGHT MAGS OFF THEN ON ONE AT A TIME FUEL PUMP ON THROTTLE ADJUST MIXTURE RICH ALT. AIR OPEN IF POWER RESTORED ALT. AIR CLOSE FUEL PUMP OFF IF POWER NOT RESTORED TRANSPONDER 7700 RADIO 121.5 RADIO FAILURE AUDIO PANEL VERIFY RADIO SELECTION MIC JACK CHECK VOLUME TEST CIRCUIT BREAKERS IN FREQUENCY CHECK AUDIO CONTROL C.B. PULL TRANSPONDER 7600 *FOLLOW LOST COMM PROCEDURES OPEN DOOR SLOW AIRPLANE TO 87 KIAS CABIN VENTS CLOSE IF UPPER LATCH IS OPEN LATCH IF SIDE LATCH IS OPEN PULL ON ARMREST WHILE CLOSING LATCH IF BOTH CLOSE SIDE, THEN TOP LATCH |  |
| University of Oklahoma School of Aviation Printed 11/18/2024 |  | 9 |  | University of Oklahoma School of Aviation Printed 11/18/2024 | 10 |
| EMERGENCY PROCEDURES PA-28-181 PIPER PILOT |  |  |  | EMERGENCY PROCEDURES PA-28-181 PIPER PILOT |  |
| ALTERNATOR FAILURE VERIFY FAILURE CHECK ALT AMPS INDICATION ALTERNATOR OFF ALT CIRCUIT BREAKER RESET IF TRIPPED ALT FIELD CIRCUIT BREAKER RESET ALTERNATOR ON IF POWER NOT RESTORED ALTERNATOR OFF ELECTRICAL LOAD SHED *LAND AS SOON AS POSSIBLE* COMPLETE ELECTRICAL FAILURE STANDBY INSTRUMENT VERIFY OPERATIONAL AIRCRAFT CONTROL USE STANDBY INSTRUMENT AIRSPEED DO NOT EXCEED 110 KIAS AFTER ENGINE SHUTDOWN STANDBY INSTRUMENT OFF (PRESS AND HOLD PWR BUTTON FOR 5 SECONDS) PFD FAILURE STANDBY INSTRUMENTS VERIFY OPERATIONAL AIRCRAFT CONTROL USE STANDBY INSTRUMENTS AIRSPEED DO NOT EXCEED 110 KIAS COM 1 (EXTERNAL COM) TUNE AS NECESSARY NAVIGATION USE G5 DISPLAY WITH GPS1 (EXTERNAL GPS) OR NAV 2 (VOR/LOC/ILS) SOURCE INFO EXIT AND AVOID IFR CONDITIONS AS SOON AS PRACTICAL. *Warning* All aural alerts, except stall warning, become inoperative when the PFD fails. Maintain additional airspeed awareness during low and high speed flight. |  |  |  | CYLINDER HEAD TEMPERATURE ENGINE PAGE SELECT CYLINDER HEAD TEMPERATURES MONITOR THROTTLE CONSIDER REDUCING MIXTURE CONSIDER ENRICHENING AIRSPEED CONSIDER INCREASING IF CYLINDER HEAD TEMPERATURE CONTINUES TO RISE THROTTLE MINIMUM REQUIRED MIXTURE RICH AIRSPEED INCREASE IF PRACTICAL CYLINDER HEAD TEMPERATURES MONITOR IF CYLINDER HEAD TEMPERATURE REMAINS IN WARNING RANGE *LAND AS SOON AS POSSIBLE* *PREPARE FOR POWER OFF LANDING* OIL PRESSURE LOW OIL PRESSURE THROTTLE MINIMUM REQUIRED IF HIGH OIL TEMP LAND AS SOON AS POSSIBLE IF NORMAL OIL TEMP LAND AS SOON AS PRACTICAL HIGH OIL PRESSURE THROTTLE MINIMUM REQUIRED LAND AS SOON AS PRACTICAL OIL TEMPERATURE THROTTLE MINIMUM REQUIRED MIXTURE FULL RICH AIRSPEED INCREASE IF PRACTICAL LAND AS SOON AS POSSIBLE SEE POH FOR OTHER EMERGENCIES OR ABNORMALITIES LISTED - LOSS OF FUEL FLOW - FUEL QUANTITY LOW - ADAHRS FAILURE - ADAHRS DEGRADED MODES - ERRONEOUS OR LOSS OF ENGINE AND FUEL DISPLAYS - LOSS OF NAVIGATION DATA - AUTOPILOT OR ESP MALFUNCTIONS - AUTOMATIC AUTOPILOT DISCONNECT - ELECTRIC PITCH TRIM FAILURE - ELECTRIC PITCH TRIM RUNAWAY - ESP ACTIVATION - AUTOPILOT OVERSPEED RECOVERY - AUTOPILOT UNDERSPEED RECOVERY - AUTOPILOT OUT-OF-TRIM - ABNORMAL FLIGHT DIRECTOR MODE TRANSITIONS - AUTOPILOT PREFLIGHT TEST FAILURE - AUTOPILOT – LOSS OF NAVIGATION INFORMATION - AUTOPILOT – LOSS OF AIRSPEED/ALTITUDE INFORMATION - AUTOPILOT – LOSS OF GPS INFORMATION - PITOT HEAT FAILURE - STARTER ENGAGED - SPIN RECOVERY |  |
| University of Oklahoma School of Aviation Printed 11/18/2024 |  | 11 |  | University of Oklahoma School of Aviation Printed 11/18/2024 | 12 |
| EMERGENCY PROCEDURES PA-28-181 PIPER PILOT |  |  |  | EMERGENCY PROCEDURES PA-28-181 PIPER PILOT |  |
|  |  |  |  | SYMPTOMS OF VAPOR LOCK - FLUCTUATION OF IDLE SPEED AND FUEL FLOW - POOR ENGINE RESPONSE TO THROTTLE MOVEMENT - ENGINE WILL NOT OPERATE WHEN THROTTLE IS CLOSED - HIGH RPM DROP (GREATER THAN 175 RPM) DURING MAG CHECK CORRECTIVE ACTION FOR VAPOR LOCK - ADVANCE THROTTLE TO ~2000 RPM FOR 1-2 MIN OR UNTIL SMOOTH ENGINE OPERATIONS - RETARD THROTTLE TO IDLE AND CHECK FOR SMOOTH OPERATION - DURING TAXI, LEAN MIXTURE AND OPERATE AT A HIGH POWER SETTING (MAX 1200 RPM) AS PRACTICAL - PRIOR TO TAKEOFF SET MIXTURE TO FULL RICH - VERIFY SMOOTH ENGINE OPERATION DURING TAKEOFF ROLL |  |
| University of Oklahoma School of Aviation Printed 11/18/2024 |  | 13 |  | University of Oklahoma School of Aviation Printed 11/18/2024 | 14 |
| COMMON FREQUENCIES |  |  |  |  |  |
| # Max Westheimer (KOUN) AWOS 119.55 Ground 121.60 Tower 118.00 LOC IPHY 111.10 ILS IBWM 111.95 OU Traffic 123.30 Cruise Aviation 122.95 # David Jay Perry (1K4) CTAF 122.7 # Chickasha (KCHK) AWOS 118.175 CTAF 123.00 # Shawnee (KSNL) AWOS 118.275 CTAF 122.70 # Will Rogers (KOKC) ATIS 125.85 Clearance Delivery 124.35 Ground 121.90 Tower 119.35 IRW 114.10 | # Okla. City Approach Control NW 124.60 NE 124.20 SW 124.60 SE 120.45 S 133.60 # Transponder Codes 7500 Hijack 7600 Lost Comm. 7700 Emergency # Contact Numbers Aviation Department 405-325-7231 Mobile Phone 405-919-6319 OU Police Department 405-325-1717 |  |  |  |  |
| University of Oklahoma School of Aviation Printed 11/18/2024 |  | 15 |  |  |  |
|  |  |  |  | Training Checkpoints - 1 = Amber - 2 = Good Life (170K) 18 DME off the IRW 203 radial - 3 = East Chickasha (watch for active parachute jumping area*) - 4 = Alexx - 5 = West Dibble – 19 DME off the IRW 220 Radial - 6 = North Dibble - 7 = South Dibble - 8 = Cole - 9 = Lindsay - 10 = Woody Chapel - 11 = Washington - 12 = Purcell* Always check NOTAM for possible sUAS activity up to 5,000 MSL - 13 = Pink - 14 = Oil Silo - 15 = Wanette - 16 = East Lake Thunderbird - 17 = South Hickory Hills - 18 = McCaslin - 19 = Wayne - 20 = Lexington * Always check Lexington NOTAM (possible sUAS activity) - 21 = Thunder Valley |  |
| University of Oklahoma School of Aviation Printed 11/18/2024 |  | 16 |  | University of Oklahoma School of Aviation Printed 11/18/2024 | 17 |